Tortricidae Molecular Protocol

T. M. Gilligan

Colorado State University

June 2010

PCR purification/cleanup using Qiagen QIAquick PCR Purification Kit:

- Add 250 µl Buffer PB to 1.5 ml tube

- Add PCR product (~50 µl) to same tube, vortex and spin down

- Add mixture to spin column (300 µl)

- Spin at 15,000 RPM for 1 minute

- Discard flow through, dry tube

- Add 750 µl Buffer PE

- Spin at 15,000 RPM for 1 minute

- Discard flow through, dry tube

- Spin at 15,000 RPM for 1 minute, discard flow through and tube

- Place spin column in clean, labeled 1.5 ml tube

- Elute DNA with 35 µl Buffer EB, spin at 15,000 RPM for 1 minute

At this point samples are ready for sequencing.

Optional:

The amount of DNA in samples can be quantified using a Low Mass Ladder on a normal

(high-melt) gel:

Ladder: 2 µl ladder + 8 µl water

DNA: 2 µl PCR product + 2 µl 6X loading dye + 6 µl water

Examine band intensity to determine quantity of DNA for each sample.